# Documentation For Robonet Project

(DEVELOPERS GUIDE)

## Requirements:

## Windows 2000 or Windows xp

Knowledge in Mfc visual c++

Knowldege in internet technologies(javascript and html)

IIS 5.0 and above (you can find the installation in the windows  2000 and windows xp disk).

Visual Studion 6.0 Environment

## Environment

This Project was developed in Visual c++ (ISAPI DLL MFC EXTENSION).

In this Environment You can write the interface to the client in Javascript, and excecute commands to the robot in MFC language.

The dll is loaded to the memory in the server and manages all the requests that the clients send to the application.

Structure of Demo:

The demo is built from one html page that consists inside 4 frames that are hidden,and through those frames the commands to the controller are sent to the robonet.dll.

The Reply of the controller about the actions that were made are sent back to the client through the dll and the main frame is updated.

Each frame has a target .

The frame that holds all the html pages inside is called  IDR_FRAMESET and it gets all the html pages from the dll,from functions that create this html.

Getmainframe In the CrobonetExtension class is responsible to build the user interface for the use of the robot. This frame is updated from the other frames.

Getpicframe In the CrobonetExtension class is responsible to  update the camera pictures all the time it is updated every 10-120 seconds  according to the adminisitrators settings.

Controlframe in the Crobot creates frame that send commands to the client,and refreshes again.

Errorframe In the Crobot class is sending the error messages from the controller ,updates the axes,updates connection status.

Getendmotion In the Crobonet class is responsible for sending the end of motion requests to the server.

The operations that are demonstrated in this demo:

- Performing Online/Offline with the controller.

- Performing ControlOff/ControlOn with the controller.

- Moving the robot manualy in joints,and xyz.

- Homing the robot.

- Activating Outputs in the controller.

- See the robots position in xyz,joints,and encoders.

How to Connect to the robot?

In order to program the controller you will need to load the usbc dll in the beginning of your application,and include all the dll files.

In this example the usbc directory is in the robonet sources directory.

The libraries that needs to be included:

"Error.h"

"Usbcdef.h"

"Usbc.h"

“Extern.h”

In this example those files were included in the stfdafx.h.

#### Functions That are used in this demo:

- Perform Online/Offline:

To Activate Online/Offline:

a. Initialization(INIT_MODE_ONLINE,rob_type, (CallBackFun)&InitEnd, (CallBackFun)&ErrorMessage);

In the first field you will write the  required mode to connect to controller:

In order to perform online:INIT_MODE_ONLINE

In order to perform online:INIT_MODE_OFFLINE

Second field you will write the robot type that you are using:

If  The  robot type is ER-2 write:21

Initialization(INIT_MODE_ONLINE,21, (CallBackFun)&InitEnd, (CallBackFun)&ErrorMessage);

If  The  robot type is ER-4 write:41

Initialization(INIT_MODE_ONLINE,41, (CallBackFun)&InitEnd, (CallBackFun)&ErrorMessage);

In the second parameter you will define call back function that will  get answer that initialization was finished.

This function will be written in the following syntax:

void  InitEnd(ConfigData *pTheConfigData)

The third parameter is the Error Message that  you can get the error number that accured and to post message to the suitable function.

void  ErrorMessage(ErrorInfo *pTheErrorInfo)

In this demo you can see the syntax in the commands in the following functions:

a.Online-Crobot Class function Online

b.Offline-Crobot Class function Offline.

- Control on/off

In order to activate  Controlon/Controloff

To excecute control on write the following syntax:

Control( '&', TRUE )

First parameter ‘&’ declares that you will perform control on for all the devices        	that are connected to controller.

To excecute control off write the following syntax:

Control( '&', FALSE )

You can see how it works in the function:Control_on that belongs to Crobot class.

- Homming robot:

In order to home the robot use the following command:

Home('A',(CallBackFun)&fnHomingNotif);

First Parameter is used to define the type of axes you want to home,

A-for robot,b-for peripherals,& for all.

The second parameter is for a call back function that declared:

void fnHomingNotif(UCHAR *ucNotifCode)

When *ucNotifCode returns immediatly the number of axes that is homed.

In this code of robonet project you can see The work with this command in function Home_robot in the Crobot class.

- Move robot manualy:

In order to move the robot You must do the following:

EnterManual( MANUAL_TYPE_ENC )

In this function you declare what type of moving you will excecute.

In This command you can see that you declare that you will perform joint movement.

If The required movement is in the  xyz  workspace you will write:

EnterManual( MANUAL_TYPE_XYZ )

After sending this command you must write the following command:

MoveManual((unsigned char)axesnum,speedval*10);

In this function The first parameter declares the number of axes you want to             move,and the second parameter declares the speed you want to move axes.

If the speedval will be negative you will move the robot to the opposite direction.

You can see the work with this commands in the Crobot class in the function: move.

When you want to finish movement you must use the following command:

CloseManual().

Note: After about one second the robot will stop if the command MoveManual was not sent again.

- OpenGripper/CloseGripper:

In order to open gripper use the command: OpenGripper.

In order to Close gripper use the command:CloseGripper.

- Outputs

In order to activate outputs use the function:

SetDigitalOutput(0,OUT_ON);

The first parameter determing what is the output number that is refered.

You can activate Outputs from 0-7.

The Second parameter indicates if the output will be turn on or not.

7.   Watch Xyz Movement,Joint movement and encoders movement.

In order to see the Movement  of the robot on the different axes

You must do the following:

To see the movement in the encoders axes use:

void ViewShowEnco(RobotData TheEnco)

void ViewWatchJoint(RobotData TheJoint)

ViewShowEnco(RobotData TheEnco)

When you don’t want that the axes  will be updated all the time you can

Use the functions:

CloseWatchJoint();

8.    Checking the online and emergency:

IsOnLineOk()-Checking if there is a connection to controller.

If it is true the controller is online else controller offline.

This function is in the controlframe function in the Crobot class.

IsEmergency()-Checks if the emergency button is pressed if it is true returns true  else returns false.

In this demo this command is in:Ronline in the crobonetextension class.