Robot Simulation Demos

System Requirements

For best performance, the following system is recommended:

- PC with Pentium 200 MHz or higher.

- 16 MB RAM or more.

- 15 MB available on hard disk.

- Windows 95, Windows 98, or Windows NT.

- Super VGA display adapter and monitor, 256 colors.

- Mouse or other pointing device.

Installing the Demo Software

To install the Simulation demo software, do the following:

1.  Start Windows.

2.  Close any applications that are open before you begin the installation.

3.  Find and run the file \ER-DEMO\SETUP.EXE.

4.  Follow the instructions that appear on the screen.

By default the Simulation software is installed in the directory C:\ER-DEMO.

The installation creates a program group with a collection of demo programs.

Double click on any icon to run a demo.

Note: There are two versions of the graphic display software which runs the demonstration programs:

- The Graphic Display version (window) runs demos of cells/ applications containing only one robot.

- The CIM Simulation version (window) runs demos of cells/ applications contains more than one robot (MicroCIM and CIM).

Running a Demo

To execute the simulation demos files, do either of the following:

- In the demo program group, click on any of the demo icons.

OR

- Select Start | Programs | ER-Demo | and select any of the demo programs.

Do NOT use the File | Open  option to load demos.

Do NOT attempt to run more than one demo program at a time.

Stopping a Demo

To stop execution of a demo, select File | Exit.

If a Scripter window is also open (CIM applications), close it.

Failure to stop and close the Scripter window may result in an system error when you attempt to load another demo program.

Viewing Options

Rotate

To rotate the scene, place the cursor anywhere in the graphic window and:

- Click the right mouse button and drag to the right to rotate the display counterclockwise.

- Click the right mouse button and drag to the left to rotate the display clockwise.

Angle

To change the angle of the overhead scene, place the cursor on the vertical scroll bar and drag it up and down.

Zoom

To zoom the scene, place the cursor anywhere in the graphic window and:

- Click the right mouse button and drag up to zoom in.

- Click the right mouse button and drag down to zoom out.

Shading

To change the shading and lighting, click and drag the right mouse button while holding down [Shift].

Menus

Note that similar functions have different menu names in the CIM Simulation and Graphic Display modules, as noted below.

File Menu

Even if available, do NOT use any option in the File menu except Exit.

CIM Simulation	Graphic Display

Exit		(Exit)

Quits the demo and closes the graphic simulation window.

View Menu

CIM Simulation	Graphic Display

Top View   	(Top View)

Displays an overhead view of the cell.

Redirect Camera   (Redirect Camera

Allows you to select a different focal point in the graphic display of the cell.  To change the center point of the graphic display window, select View|Redirect Camera. Then click on any point in the scene. It now becomes the center point of the graphic display.

Follow Me Camera   (Follow Me Camera)

When selected, the camera follows a specific focal point. This function is Similar to Redirect Camera, only automatic and continuous. To change the focal point of the camera, you must first clear this menu option to stop the  camera's tracking. Then select

Follow Me Camera again, and select another focal point. This function is particularly useful for following the motions of the robot gripper. But it can also be used to track any object in the cell that is being moved.

Save Layout Restore Layout	(Save Camera Position)

Saves the current view of the cell. The graphic display will show this view whenever you select Cell Reset or Restore Camera Position, or when the cell setup file is loaded.

Always on Top	   (Always on Top)

Keeps the Graphic Display window in front of all other windows and dialog boxes.

Robot Menu

CIM Simulation	Graphic Display

(NA)		(Stop    	F9)

Halts execution of the demo program.

Show Menu

CIM Simulation	Graphic Display

(NA)		Show Robot Path

When selected, dots showing the path of the gripper will be drawn on the screen whenever the robot moves. Each dot is a cube with a different colored side, which allows you to determine the orientation of the TCP at each point. The path is drawn at a rate of 3 dots per second. After 500 dots are drawn, all dots are cleared. Clear this menu option to stop the display of the robot path.

(NA)		Clear Robot Path

Removes the robot path that was drawn on the screen.

(NA)		Show Gripper Position

Displays in the Graphic Display window's status line the robot's current Cartesian (XYZ) position. The values indicate the position of the tip of the gripper relative to the robot's point of origin.

(NA)		Show Cell Origin

The cell has a point of origin (0,0) at table/pedestal level. The positions of all objects are defined as relative to this point of origin.  When Show|Show Cell Origin is selected, a red cross marked with X+ and Y+ axes remains displayed on the screen. When manipulating the graphic display, the cross may disappear momentarily.  Rotating the view does not change the X and Y dimensions of the cell.

(NA)		Show Robot Work Envelope

Displays the span of the robot's working range, and allows you to see whether objects are within the robot's reach.

Text Menu    –    Labels Menu

CIM Simulation	Graphic Display

Show None	Hide All Labels

Hides the display of all labels.

Show Names	Object Names

When selected, a label on each object shows its name, as defined in the object's properties menu.

(NA)		Object Positions

When selected, a label on each object shows its position. The coordinates indicate the center point of the object relative to the cell's point of origin.  These coordinates are displayed in black text.

(NA)		Object Positions in Robot Coordinates

When selected, the coordinates shown in the labels indicate the objects' positions relative to the robot's center of origin.  If the robot has been moved on a slidebase, or repositioned during setup, these coordinates provide the proper coordinates needed for teaching robot positions. These coordinates are displayed in blue text.

Show Ext ID	(NA)

Displays an external identification number.

Show Palettes	  (NA)

Displays the identification numbers of pallets in the CIM cell.

Show Templates (NA)

Displays the identification numbers of templates in the CIM cell.

Show Parts	(NA)

Displays the identification numbers of parts in the CIM cell.

website: www.eshed.com

email: info@eshed.com